服务器接入路由器

Router(config)#int f0/0

Router(config-if)#ip add 192.168.8.5 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int f0/1

Router(config-if)#ip add 192.168.9.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#ip helper-address 192.168.8.3

Router(config-if)#ex

Router(config)#router rip

Router(config-router)#v 2

Router(config-router)#no au

Router(config-router)#net 192.168.8.0

Router(config-router)#net 192.168.9.0

出口路由器

Router(config)#int f0/0

Router(config-if)#ip add 192.168.9.2 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int f0/1

Router(config-if)#ip add 192.168.10.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int s0/3/0

Router(config-if)#ip add 100.1.1.1 255.255.255.0

Router(config-if)#clock rate 64000

Router(config-if)#no sh

Router(config-if)#ex

Router(config)#int f0/0

Router(config-if)#ip nat inside

Router(config-if)#int f0/1

Router(config-if)#ip nat inside

Router(config-if)#int s0/3/0

Router(config-if)#ip nat outside

Router(config-if)#ex

Router(config)#ip nat pool DZC 100.1.1.3 100.1.1.10 netmask 255.255.255.0

Router(config)#access-list 1 permit 192.168.0.0 0.0.255.255

Router(config)#ip nat inside source list 1 pool DZC

Router(config)#router rip

Router(config-router)#v 2

Router(config-router)#no au

Router(config-router)#net 192.168.9.0

Router(config-router)#net 192.168.10.0

Router(config-router)#default-information originate

Router(config-router)#ex

Router(config)#ip route 0.0.0.0 0.0.0.0 100.1.1.2

核心路由器

Router(config)#int f0/0

Router(config-if)#ip add 192.168.10.2 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int f0/1

Router(config-if)#ip add 192.168.12.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#ip helper-address 192.168.8.3

Router(config-if)#int f1/0

Router(config-if)#ip add 192.168.13.1 255.255.255.0

Router(config-if)#ip helper-address 192.168.8.3

Router(config-if)#no sh

Router(config-if)#int s0/3/0

Router(config-if)#ip add 192.168.11.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#ip helper-address 192.168.8.3

Router(config-if)#ex

Router(config)#router rip

Router(config-router)#v 2

Router(config-router)#no au

Router(config-router)#net 192.168.10.0

Router(config-router)#net 192.168.11.0

Router(config-router)#net 192.168.12.0

Router(config-router)#net 192.168.13.0

四牌楼接入路由器

Router(config)#int f0/0

Router(config-if)#ip add 192.168.7.254 255.255.255.0

Router(config-if)#no sh

Router(config-if)#ip helper-address 192.168.8.3

Router(config-if)#int s0/3/0

Router(config-if)#ip add 192.168.11.2 255.255.255.0

Router(config-if)#clock rate 64000

Router(config-if)#no sh

Router(config-if)#ex

Router(config)#router rip

Router(config-router)#v 2

Router(config-router)#no au

Router(config-router)#net 192.168.7.0

Router(config-router)#net 192.168.11.0

核心交换机1

Switch(config-vlan)#vlan 60

Switch(config-if)#int vlan 60

Switch(config-if)#ip add 192.168.6.254 255.255.255.0

Switch(config-if)#ip helper-address 192.168.8.3

Switch(config-if)#ex

Switch(config-if)#int f0/7

Switch(config-if)#sw mo access

Switch(config-if)#sw ac vlan 60

Switch(config-if)#ex

interface range f0/1-5

no switchport

exit

interface f0/1

description to-Dist0

ip address 10.0.10.2 255.255.255.252

exit

interface f0/2

description to-Dist1

ip address 10.0.20.2 255.255.255.252

exit

interface f0/3

description to-Dist2

ip address 10.0.30.2 255.255.255.252

exit

interface f0/4

description to-Dist3

ip address 10.0.40.2 255.255.255.252

exit

interface f0/5

description to-Dist4

ip address 10.0.50.2 255.255.255.252

exit

router rip

network 10.0.10.0

network 10.0.20.0

network 10.0.30.0

network 10.0.40.0

network 10.0.50.0

exit

核心交换机2与核心交换机1同理，将VLAN60变为VLAN90

Switch(config-if)#int vlan 90

Switch(config-if)#standby 10 ip 192.168.90.254

Switch0

Switch(config)#vlan 10

Switch(config-vlan)#exit

Switch(config)#int f0/1

Switch(config-if)#sw mo tr

Switch(config-if)#switchport trunk native vlan 99

Switch(config-if)#int f0/4

Switch(config-if)#sw mo tr

Switch(config-if)#int f0/2

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 10

Switch(config-if)#int f0/3

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 10

Switch1

Switch(config)#vlan 20

Switch(config-vlan)#ex

Switch(config)#int f0/1

Switch(config-if)#sw mo tr

Switch(config-if)#switchport trunk native vlan 99

Switch(config-if)#int f0/4

Switch(config-if)#sw mo tr

Switch(config-if)#int f0/2

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 20

Switch(config-if)#int f0/3

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 20

Switch2

Switch(config)#int f0/1

Switch(config-if)#sw mo tr

Switch(config-if)#switchport trunk native vlan 99

Switch(config-if)#int f0/4

Switch(config-if)#sw mo tr

Switch(config-if)#int f0/2

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 30

Switch(config-if)#int f0/3

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 30

Switch(config-if)#ex

Switch(config)#vlan 30

Switch3

Switch(config)#int f0/1

Switch(config-if)#sw mo tr

Switch(config-if)#switchport trunk native vlan 99

Switch(config-if)#int f0/4

Switch(config-if)#sw mo tr

Switch(config-if)#int f0/2

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 40

Switch(config-if)#int f0/3

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 40

Switch(config-if)#ex

Switch(config)#vlan 40

Switch4

Switch(config)#vlan 50

Switch(config-vlan)#ex

Switch(config)#int f0/1

Switch(config-if)#sw mo tr

Switch(config-if)#switchport trunk native vlan 99

Switch(config-if)#int f0/4

Switch(config-if)#sw mo tr

Switch(config-if)#int f0/2

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 50

Switch(config-if)#int f0/3

Switch(config-if)#sw mo ac

Switch(config-if)#sw ac vlan 50

外网路由器

Router(config)#int f0/0

Router(config-if)#ip add 100.1.2.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int f0/1

Router(config-if)#ip add 100.1.3.1 255.255.255.0

Router(config-if)#no sh

Router(config-if)#int s0/3/0

Router(config-if)#ip add 100.1.1.2 255.255.255.0

Router(config-if)#no sh

Dist0交换机

enable

configure terminal

hostname Dist0

! 创建VLAN

vlan 10

exit

interface f0/1

description to-Switch0

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10

exit

interface g0/1

description to-Core1

no switchport

ip address 10.0.10.1 255.255.255.252

exit

interface g0/2

description to-Core2

no switchport

ip address 10.0.10.5 255.255.255.252

exit

interface vlan 10

ip address 192.168.1.254 255.255.255.0

standby 10 ip 192.168.1.253

standby 10 priority 110

standby 10 preempt

exit

ip routing

router rip

version 2

network 192.168.1.0

network 10.0.10.0

no auto-summary

exit

Dist1交换机

enable

configure terminal

hostname Dist1

! 创建VLAN

vlan 20

exit

interface f0/1

description to-Switch1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20

exit

interface g0/1

description to-Core1

no switchport

ip address 10.0.20.1 255.255.255.252

exit

interface g0/2

description to-Core2

no switchport

ip address 10.0.20.5 255.255.255.252

exit

interface vlan 20

ip address 192.168.2.254 255.255.255.0

standby 20 ip 192.168.2.253

standby 20 priority 110

standby 20 preempt

exit

ip routing

router rip

version 2

network 192.168.2.0

network 10.0.20.0

no auto-summary

exit

Dist2交换机

enable

configure terminal

hostname Dist2

! 创建VLAN

vlan 30

exit

interface f0/1

description to-Switch2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30

exit

interface g0/1

description to-Core1

no switchport

ip address 10.0.30.1 255.255.255.252

exit

interface g0/2

description to-Core2

no switchport

ip address 10.0.30.5 255.255.255.252

exit

interface vlan 30

ip address 192.168.3.254 255.255.255.0

standby 30 ip 192.168.3.253

standby 30 priority 50

standby 30 preempt

exit

ip routing

router rip

version 2

network 192.168.3.0

network 10.0.30.0

no auto-summary

exit

Dist3交换机

enable

configure terminal

hostname Dist3

! 创建VLAN

vlan 40

exit

interface f0/1

description to-Switch3

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 40

exit

interface g0/1

description to-Core1

no switchport

ip address 10.0.40.1 255.255.255.252

exit

interface g0/2

description to-Core2

no switchport

ip address 10.0.40.5 255.255.255.252

exit

interface vlan 40

ip address 192.168.4.254 255.255.255.0

standby 40 ip 192.168.4.253

standby 40 priority 50

standby 40 preempt

exit

ip routing

router rip

version 2

network 192.168.4.0

network 10.0.40.0

no auto-summary

exit

Dist4交换机

enable

configure terminal

hostname Dist4

! 创建VLAN

vlan 50

exit

interface f0/1

description to-Switch4

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50

exit

interface g0/1

description to-Core1

no switchport

ip address 10.0.50.1 255.255.255.252

exit

interface g0/2

description to-Core2

no switchport

ip address 10.0.50.5 255.255.255.252

exit

interface vlan 50

ip address 192.168.5.254 255.255.255.0

standby 50 ip 192.168.5.253

standby 50 priority 50

standby 50 preempt

exit

ip routing

router rip

version 2

network 192.168.5.0

network 10.0.50.0

no auto-summary

exit

ip access-list extended IOT_POLICY

permit tcp any host 192.168.90.1 eq 80

deny ip any 192.168.90.0 0.0.0.255

permit ip any any

exit

interface Vlan80

ip access-group IOT_POLICY in

exit

使用三层架构

优势：网络架构优化，性能提升，管理与维护，简化安全增强，可扩展性与灵活性，路由优化，成本效益，服务质量(QoS)优化，监控与排障，冗余与高可用性。

运用的协议：

NAT，RIP，ACL，ARP，HSRP，VLAN